# LogicPad Pinout and Hardware Configuration

Microcontroller: STM32F103C8 (Cortex-M3)

Project Name: LogicPad (9-Key Macro Keyboard with OLED Display)

This document details the complete pin assignment and interfacing for all external components connected to the STM32F103C8, including the key matrix, RGB multiplexing system, OLED display, and debugging interface.

## 1. System Core and Debugging

| Function | STM32 Pin | Port/Pin | Notes |
| --- | --- | --- | --- |
| External Clock (HSE) | OSC_IN | PD0 | External 8 MHz Crystal (Atypical for standard blue-pill, but per specification) |
| External Clock (HSE) | OSC_OUT | PD1 | External 8 MHz Crystal (Atypical for standard blue-pill, but per specification) |
| SWD Debugging Clock | SWCLK | PA14 | Serial Wire Debug Clock |
| SWD Debugging Data | SWDIO | PA13 | Serial Wire Debug I/O |

## 2. Key Matrix Pinout (3x3 Array)

The 9 primary keys are connected in a 3x3 matrix, utilizing 3 column outputs and 3 row inputs. The row inputs should be configured with internal Pull-Up resistors.

| Matrix Role | Key Role | STM32 Port/Pin | I/O Type |
| --- | --- | --- | --- |
| Column 0 | C0_OUT | PB5 | Output (Active Low/High Drive) |
| Column 1 | C1_OUT | PB4 | Output (Active Low/High Drive) |
| Column 2 | C2_OUT | PB3 | Output (Active Low/High Drive) |
| Row 0 | R0_IN | PB12 | Input (Pull-Up) |
| Row 1 | R1_IN | PB13 | Input (Pull-Up) |
| Row 2 | R2_IN | PB14 | Input (Pull-Up) |

### 2.1 Dedicated Button

This is a standalone button, separate from the 3x3 matrix.

| Key Role | STM32 Port/Pin | Wiring | I/O Type |
| --- | --- | --- | --- |
| Dedicated Key | PB15 | Connected to GND via the button switch | Input (Pull-Up Required) |

## 3. RGB LED Multiplexing Pinout

This system uses multiplexing (or Charlieplexing logic) to control the color and state of 10 individual RGB LEDs (9 matrix keys + 1 dedicated key). PB8, PB9, and PB10 provide the common color control (R, G, B) for all LEDs, while PA0-PA9 select which individual LED is illuminated. These color control pins should be driven using PWM for intensity control.

| LED Function | STM32 Pin | Purpose | Notes |
| --- | --- | --- | --- |
| Red Color Control | PB8 (TIM4_CH3) | Common Red Channel | PWM Recommended |
| Green Color Control | PB9 (TIM4_CH4) | Common Green Channel | PWM Recommended |
| Blue Color Control | PB10 (TIM2_CH3) | Common Blue Channel | PWM Recommended |
| C0R0 LED Select | PA0 | Individual LED Enable / Sink | Key (0, 0) |
| C0R1 LED Select | PA1 | Individual LED Enable / Sink | Key (0, 1) |
| C0R2 LED Select | PA2 | Individual LED Enable / Sink | Key (0, 2) |
| C1R0 LED Select | PA3 | Individual LED Enable / Sink | Key (1, 0) |
| C1R1 LED Select | PA4 | Individual LED Enable / Sink | Key (1, 1) |
| C1R2 LED Select | PA5 | Individual LED Enable / Sink | Key (1, 2) |
| C2R0 LED Select | PA6 | Individual LED Enable / Sink | Key (2, 0) |
| C2R1 LED Select | PA7 | Individual LED Enable / Sink | Key (2, 1) |
| C2R2 LED Select | PA8 | Individual LED Enable / Sink | Key (2, 2) |
| Dedicated Key LED Select | PA9 (TIM1_CH2) | Individual LED Enable / Sink | Dedicated Key |

## 4. OLED Display Pinout (I2C)

A 0.96-inch OLED display is connected using the I2C communication protocol.

| I2C Function | STM32 Pin | Notes |
| --- | --- | --- |
| I2C1 SCL (Clock) | PB6 | Requires external Pull-Up resistor |
| I2C1 SDA (Data) | PB7 | Requires external Pull-Up resistor |

## 5. Summary Pinout Table

| Port/Pin | Primary Function | Secondary Function |
| --- | --- | --- |
| PA0 | C0R0 LED Select |  |
| PA1 | C0R1 LED Select |  |
| PA2 | C0R2 LED Select |  |
| PA3 | C1R0 LED Select |  |
| PA4 | C1R1 LED Select |  |
| PA5 | C1R2 LED Select |  |
| PA6 | C2R0 LED Select |  |
| PA7 | C2R1 LED Select |  |
| PA8 | C2R2 LED Select |  |
| PA9 | Dedicated LED Select (TIM1_CH2) |  |
| PA13 | SWDIO | Debugging Data |
| PA14 | SWCLK | Debugging Clock |
| PB3 | Column 2 Output |  |
| PB4 | Column 1 Output |  |
| PB5 | Column 0 Output |  |
| PB6 | I2C1 SCL | OLED Clock |
| PB7 | I2C1 SDA | OLED Data |
| PB8 | R_Ctrl (TIM4_CH3) | RGB Common Red (PWM) |
| PB9 | G_Ctrl (TIM4_CH4) | RGB Common Green (PWM) |
| PB10 | B_Ctrl (TIM2_CH3) | RGB Common Blue (PWM) |
| PB12 | Row 0 Input |  |
| PB13 | Row 1 Input |  |
| PB14 | Row 2 Input |  |
| PB15 | Dedicated Button Input |  |
| PD0 | External Clock Input (HSE) | Based on user specification |
| PD1 | External Clock Output (HSE) | Based on user specification |